BCI Arm Motor Imagery Copilot

Progress Report — June 29, 2026

# 1. Goal

Construct the copilot pipeline that maximizes BCI+Copilot classification accuracy on the 8-direction cursor control task. The copilot is a learned module that augments BCI-decoded cursor velocity in real time; its quality depends on several interacting design choices. The pipeline dimensions under consideration are:

Default Parameters

| Dimension | Options | Values |
| --- | --- | --- |
| Model Type | 2 | SL, RL |
| Data Source | 4 | Old decoder, EEGK sim, EEGK real, EEGK surrogate |
| Copilot Velocity Magnitude Set | 3 | Low (0.01), default (0.02), high (0.03) normalized units/tick Copilot vel magnitude = 1 / number of ticks per trial |
| Input Feature Set Fine Tune | 2 | Current (position, unit velocity, velocity magnitude) / Extended (+ raw BCI cursor position) |
| Tick Weighting Fine Tune | 3 | Constant, linear, exponential |
| Performance Metric Fine Tune | 2×2 | SL: classification accuracy, angle-error minimizing RL: linear distance, angular distance |
| Train/Test Split Set | 3 | Cross-subject, all-subject, within-subject |

All pipeline dimensions interact through the Phase 2 DAgger augmentation loop: data source sets the raw BCI signal, copilot velocity magnitude scales the correction applied at each tick, feature set determines what the model observes at each tick, tick weighting shapes the gradient signal across ticks, and performance metric defines the loss objective. These five dimensions are therefore optimized sequentially in order of expected impact. Train/test split is the only dimension that does not interact with the others through the training loop, and is optimized last.

A fully factorial search over all permutations would require a very large number of training runs. Many permutations are invalid or redundant (e.g. RL training on labelled real human data), but the full factorial remains costly. A sequential greedy algorithm is used instead.

# 2. Sequential Greedy Optimization

The sequential greedy algorithm optimises one pipeline dimension at a time, holding all others fixed, in decreasing order of expected impact on model performance:

- Data source + model type — dominant: determines the quality, distribution, and volume of the training signal. The raw BCI accuracy gap between decoders alone (~13pp) makes this the highest-impact decision.

- Copilot velocity magnitude — directly scales every correction in Phase 2, changing every subsequent cursor position the model sees. Has an immediate and large effect on the augmented trajectory distribution across all epochs.

- Input feature set — changes what information the model has access to at each tick. Adds to what the model observes without changing the trajectory itself, so lower impact than magnitude.

- Tick weighting — shapes the gradient signal distribution across ticks. Operates on top of a fixed trajectory and feature set, so lower impact than the representation-level decisions above.

- Train/test split — affects generalisation strategy rather than raw model performance. The only dimension genuinely independent of the training loop.

- Performance metric — SL loss / RL reward formulation. Lowest marginal impact given the high correlation between classification accuracy and angle error on this task, and highest implementation cost.

Model type is jointly optimized with data source in Step 1 rather than as a separate dimension. All real human data sources (Old decoder, EEGK sim, EEGK real) favour a supervised learning approach because ground-truth target labels are available. RL is therefore only evaluated on the EEGK surrogate, where the absence of a direct supervised signal makes a reward-based approach worth comparing.

To guard against the independence assumption of the sequential greedy approach, an explicit interaction check (Step 5) re-evaluates the second-best data source from Step 1 under the winning magnitude, feature set, and tick weighting before committing to the train/test split optimization. If the ranking reverses, the interaction is non-negligible and additional runs are required.

Step 0 — Anchor (from last week’s report)

The V3 supervised LSTM copilot, trained on old decoder data with linear tick weighting and a cross-subject split, serves as the anchor for the greedy search. Note: V3 used a cardinal-direction corrective velocity (see Section 4 for the corrected formulation used in all greedy runs). V3 also used the current feature set and a default velocity magnitude of 0.02.

| Model Type | Data Source | Vel. Magnitude | Feature Set | Tick Weighting | Train/Test Split | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SL | Old decoder | 0.02 | Current | Linear | Cross-subject | 47.8% | 32.12° |

Data source baseline accuracy: 46.7%    |    Baseline angle error: 32.65°

Delta classification accuracy: +1.07pp    |    Delta angle error: +0.53° (positive = improvement)

Step 1 — Find optimal data source + model type

All other dimensions held fixed: SL model type (except 1E), default copilot velocity magnitude (0.02), current feature set, exponential tick weighting, cross-subject split. Training data is varied across five configurations.

| Config | Data Source + Model Type | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- |
| 1A | Old decoder real + SL | — | — |
| 1B | EEGK sim + SL Train - High Priority | — | — |
| 1C | EEGK real + SL Use for Eval | — | — |
| 1D | EEGK surrogate + SL Train - High Priority | — | — |
| 1E | EEGK surrogate + RL | — | — |
| 1F | EEGK sim + surrogate Blend Train -High Priority |  |  |

Table 1. Step 1 results. Winner = highest BCI+Copilot Accuracy (D*).

Winner of Step 1 = D* (best data source + model type)

Step 2 — Find optimal copilot velocity magnitude

D* held fixed alongside all other non-varying dimensions. Copilot velocity magnitude is varied across three values. The default magnitude result (2B) uses the same value as V3 and can be pulled from Step 1 if D* is the old decoder; otherwise it is a new run.

| Config | Copilot Velocity Magnitude | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- |
| 2A | Low (0.01) | — | — |
| 2B | Default (0.02) | — | — |
| 2C | High (0.03) | — | — |

Table 2. Step 2 results. Winner = highest BCI+Copilot Accuracy (M*).

Winner of Step 2 = M* (best copilot velocity magnitude)

Step 3 — Find optimal input feature set

D* and M* held fixed. The input feature set is varied between the current 5-feature input and the extended set which adds raw BCI cursor position as a sixth and seventh feature — the cursor position the BCI signal alone would have produced, excluding all prior copilot corrections.

| Config | Input Feature Set | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- |
| 3A | Current (position, unit velocity, velocity magnitude) | — | — |
| 3B | Extended (+ raw BCI cursor position) | — | — |

Table 3. Step 3 results. Winner = highest BCI+Copilot Accuracy (F*).

Winner of Step 3 = F* (best input feature set)

Step 4 — Find optimal tick weighting

D*, M*, and F* held fixed. Tick weighting scheme is varied. The exponential result (4C) uses the same setting as Step 1 and is pulled directly from there.

| Config | Tick Weighting | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- |
| 4A | Constant | — | — |
| 4B | Linear | — | — |
| 4C / 1_ | Exponential | Pull from Step 1 | Pull from Step 1 |

Table 4. Step 4 results. Winner = highest BCI+Copilot Accuracy (W*).

Winner of Step 4 = W* (best tick weighting)

Step 5 — Interaction check

To guard against the independence assumption of the sequential greedy search, the second-best data source + model type from Step 1 is re-evaluated using M*, F*, and W*. If the ranking reverses, the interaction between the top dimensions is non-negligible and additional runs are required. If the ranking holds, the greedy optimum is confirmed.

| Config | Data Source + Model Type | Tick Weighting | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- | --- |
| 5A / 4_ | 1st Best (D*) | W* | Pull from Step 4 | Pull from Step 4 |
| 5B | 2nd Best | W* | — | — |

Table 5. Interaction check. If 5B outperforms 5A, ranking has flipped and further investigation is needed.

Winner of Step 5 = Final best data source + model type + magnitude + feature set + tick weighting combination

Step 6 — Find optimal train/test split

The Step 5 winner (D* × M* × F* × W*) is held fixed. Train/test split is varied. The cross-subject result (6A) is pulled from Step 5. Within-subject (6C) will be skipped if EEGK real data is the winning source and per-subject trial counts are insufficient.

| Config | Train/Test Split | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- |
| 6A / 5_ | Cross-subject | Pull from Step 5 | Pull from Step 5 |
| 6B | All-subject | — | — |
| 6C | Within-subject | — | — |

Table 6. Step 6 results. Winner = highest BCI+Copilot Accuracy.

Step 7 — Find optimal performance metric

The Step 6 winner is held fixed. The loss function (SL) or reward function (RL) formulation is varied. The current metric result (7A) is pulled from Step 6.

| Config | Performance Metric (SL / RL) | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- |
| 7A / 6_ | Classification accuracy / Angular distance | Pull from Step 6 | Pull from Step 6 |
| 7B | Angle-error minimizing / Linear distance | — | — |

Table 7. Step 7 results. Winner = highest BCI+Copilot Accuracy — final pipeline configuration.

# 3. Best Pipeline Configuration

Summary of the pipeline configuration yielded by the sequential greedy algorithm.

| Model Type | Data Source | Vel. Magnitude | Feature Set | Tick Weighting | Train/Test Split | BCI+Copilot Accuracy | BCI+Copilot Angle Error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — | — | — | — | — | — | — | — |

Data source baseline accuracy: —    |    Baseline angle error: —

Delta classification accuracy: —    |    Delta angle error: —

# 4. Pipeline Dimension Detail

## Data Sources

- Old decoder: online_arm_trajectories.csv — 7 subjects (S01–S07), 11,760 trials, 8 directions balanced

- EEGK sim: online_arm_trajectories_EEGK_simulation.csv — same 7 subjects and trial structure as old decoder; raw EEG signals re-decoded through the EEGK decoder offline

- EEGK real: data/OnlineArmTrajectoryEEGK/ — 5 subjects (S01, S02, S04, S05, S07), real human trials recorded with the EEGK decoder directly

- EEGK surrogate: not yet implemented — requires surrogate generation script calibrated to EEGK real trajectory statistics

  - Subject specific parameters

## SL Model Architecture

Stage 1 — LSTM Target Classifier

A two-layer LSTM (hidden size 64, 51,976 parameters) processes the cursor trajectory tick-by-tick and outputs a probability distribution over the 8 target directions. The LSTM hidden state is reset to zeros at the start of each trial. Current input at each tick t:

- cursor_x, cursor_y — actual cursor position normalised by radius (432 px); includes prior copilot corrections

- vx_unit, vy_unit — unit-normalised BCI decoder velocity direction

- vel_mag_scaled — z-scored velocity magnitude (per-data-source mean and std)

Extended feature set (Step 3) additionally includes:

- bci_cursor_x, bci_cursor_y — raw BCI cursor position: where the cursor would be if no copilot corrections had been applied. Unlike (cursor_x, cursor_y), this is uncontaminated by the copilot’s own prior actions, giving the model a direct read on what the BCI decoder alone has been pushing toward.

Stage 2 — Position-Aware Corrective Velocity

Given the classifier’s predicted label and confidence at tick t, the corrective velocity is computed analytically:

- confidence = max(softmax(logits))

- direction = normalise(target_pos − cursor_pos),  where target_pos = LABEL_TO_DIR[pred_label]

- correction = direction × COPILOT_VEL × confidence,  COPILOT_VEL = M* (optimised in Step 2; default 0.02)

Implementation note: V3 (Step 0 anchor) used correction = LABEL_TO_DIR[pred] × COPILOT_VEL × confidence, which applies a fixed cardinal-direction nudge regardless of cursor position. This is only correct when the cursor is at the origin. All greedy search runs (1A onward) use the position-aware formulation above, which points the correction from the current cursor position toward the predicted target.

The confidence weighting is critical: when the classifier is uncertain (e.g. early-trial ticks where trajectory shape is ambiguous), the correction shrinks toward zero, preventing the copilot from strongly pushing in the wrong direction.

## SL Training Procedure

Data Split

Subject-level train/test split: S01–S05 (train) / S06–S07 (test) for old decoder and EEGK sim. For EEGK real and EEGK surrogate: S01, S02, S04 (train) / S05, S07 (test). This tests generalisation to completely unseen subjects.

Two-Phase DAgger

- Phase 1 (epochs 1–3): train on raw BCI-only trajectories from the CSV. The classifier has no useful signal yet; raw data avoids injecting noise from random corrections.

- Phase 2 (epochs 4–28, 25 DAgger epochs): train on augmented BCI+Copilot trajectories. At each tick the running classifier predicts a direction, applies the position-aware correction scaled by M*, and the updated cursor position is fed as input to the next tick. Augmented sequences are regenerated from scratch each epoch using the current model, so the training distribution shifts as the model improves.

- The ground-truth label at every tick is the trial target label — the same label is applied across all ticks within a trial.

Loss and Optimizer

- Loss: cross-entropy over all valid ticks, weighted by the tick weighting scheme, averaged over the batch

- Optimiser: Adam, learning rate 1e-3

- Learning rate schedule: step decay by factor 0.5 every 6 epochs

- Gradient clipping: L2 norm clipped at 1.0

- Batch size: 128

## RL Model Architecture

Policy Network

An LSTM recurrent policy network maps per-tick observations to a 9-dimensional action vector. Input per tick t: cursor position (px, py) and BCI-decoded velocity (vx, vy) — 4-dimensional input. The LSTM hidden state is reset to zeros at the start of each trial. A separate value network V(s) estimates expected reward-to-go for PPO updates.

Charge-Based Action Mechanism

- Policy output: 9-dimensional action vector at ∈ [−1, 1]⁹, one element per candidate target (8 radial + 1 centre), mapped to charge ci ∈ [0, 1]

- Force on cursor from target i: Fᴵ = K × q × cᴵ / (‖pᴵtarget − pcursor‖² + ε)

- Copilot velocity: vᶜᵖ = Σ Fᴵ × (pᴵtarget − pcursor)  summed over all 9 targets

- Final cursor velocity: 0.6 × BCI velocity + 0.3 × copilot velocity  (Lee et al. mixing coefficients)

## RL Training Procedure

Surrogate-Based Training

The RL copilot is trained entirely on surrogate-generated BCI trajectories. The surrogate policy models four features of human BCI control:

- Intended velocity: cursor moves toward the correct target at 0.5 normalised units/tick

- Direction-switch delay: drawn from U[300ms, 600ms] with linear velocity interpolation across the switch interval

- Angular noise on new decoded directions: θ ~ N(0, π/3)

- Additive Gaussian noise at every tick: ℓ ~ N(0, 0.03×I)

Reward and PPO Hyperparameters

- Reward per tick: rₜ = k × (dₜ₋₁ − dₜ), k = 66.67, dₜ = Euclidean distance from cursor to target

- Discount factor: γ = 0.98901

- Learning rate: 3×10⁻⁴  |  Rollout buffer: 2,048 steps  |  Batch size: 512  |  Total steps: 1,200,000

## Copilot Velocity Magnitude

- Low:     COPILOT_VEL = 0.01 normalised units/tick (~24% of mean old decoder BCI velocity)

- Default: COPILOT_VEL = 0.02 normalised units/tick (~47% of mean old decoder BCI velocity) — used in V3

- High:    COPILOT_VEL = 0.03 normalised units/tick (~71% of mean old decoder BCI velocity)

Mean BCI velocity differs across data sources (old decoder: 0.0424, EEGK sim: 0.0465, EEGK real: 0.0367), so the proportional strength of a fixed magnitude varies by decoder. The optimal magnitude is therefore expected to depend on the winning data source.

## Tick Weighting

- Constant:     w = [1, 1, ..., 1]

- Linear:       wₜ = t/T  for t = 1, ..., T

- Exponential:  wₜ = exp(k × (t−1)/(T−1))  for t = 1, ..., T, normalised to sum to T

## Train/Test Split

- Cross-subject — Old decoder / EEGK sim (7 subjects, S01–S07):

  - Train: S01–S05

  - Test:  S06, S07

- Cross-subject — EEGK real / EEGK surrogate (5 subjects: S01, S02, S04, S05, S07):

  - Train: S01, S02, S04

  - Test:  S05, S07

- All-subject: train and test on all available subjects; no held-out subjects

- Within-subject: train and test on a single subject; report average performance across all subjects

# 5. Possible Next Directions

- Run fully factorial search, or manually evaluate specific configurations excluded from the sequential greedy search — based on supervisor feedback

- Consider additional pipeline dimensions, for example:

  - Number of LSTM layers and hidden size — the current architecture (2 layers, hidden 64) was not explicitly optimised

  - DAgger phase ratio — the current 3/25 Phase 1/Phase 2 split was not searched

  - Ensemble of SL and RL copilots — prior results show complementary directional profiles (RL improves N/S, SL improves NE/E/W); a weighted ensemble may cover all 8 directions without a consistent loser

- Online adaptation — the copilot is currently trained offline and deployed as a fixed model. BCI decoders drift across sessions and subjects develop compensatory strategies over time. Re-training or fine-tuning the copilot after a short calibration block at the start of each session would allow it to track these shifts. Since the SL copilot trains in under 10 minutes, session-level adaptation is practically feasible and is likely the highest-impact improvement not currently in scope