Arm-BCI Copilot — Progress Report

Optimizing the copilot's training data: a leakage-free, budget-controlled re-analysis

# Overview

- Correction of prior results. The previous evaluation tested partly on training data (held-out blocks overlapped training blocks). Re-running with a leakage-free held-out split, and controlling training budget and feature normalization, changes both the baseline and the conclusion.

- Corrected baseline. A copilot trained on a subject's own real data improves accuracy by +1.25 ± 0.33 pp over raw BCI (leakage-free, 3 seeds) — down from the +1.84 pp reported under the leaky evaluation.

- Result reverses: a low sim fraction helps. Blending 25% calibrated EEGK-sim into real (real75_sim25) gives +2.14 ± 0.24 pp, beating real-only by +0.89 ± 0.14 pp — consistently across all 3 seeds and robust to a normalization confound. Higher sim fractions degrade; pure sim is worst.

- Surrogate likely not worth building. A residual-error diagnostic shows the errors the blend still makes are ordinary hard trials, not the structured/chaotic trials a synthetic surrogate could target. ~87% of BCI errors are uncorrectable from the trajectory, bounding all copilots.

# 1. Goal and fixed configuration

Goal. Find the training-data configuration that maximizes within-subject copilot accuracy on held-out real EEGK data. Everything except the training data is held fixed:

- Model: supervised LSTM copilot (2-layer, hidden 64), DAgger training.

- Corrective-velocity magnitude: 0.02 (fixed); velocity mode additive.

- Protocol: within-subject — one model per subject.

- Evaluation: held-out real EEGK test split from the same seeded partition used to build training data (leakage-free).

- Normalization: the copilot's speed feature (per-tick velocity magnitude) is rescaled to a standard range by subtracting a mean and dividing by a standard deviation, so the network sees it on a consistent scale. Those two constants are computed once from real data and reused for every blend, so the feature scale is held fixed and only data composition varies across ratios.

- Budget: fixed per subject (= that subject's real training-pool size), so every blend ratio trains on the same total with no oversampling.

- Subjects: S01, S02, S04, S05, S07 (5). S03/S06 have no real EEGK data and are excluded. Directions: 8 (N/S/E/W and diagonals).

# 2. Corrections since the last report

Four measurement issues were identified and fixed. Each was tested in isolation; the headline result survives all four.

(a) Evaluation leakage (primary). The evaluator tested on all of each subject's real data minus a calibration run; 5 of 6 data blocks per subject overlapped the blocks used for training. Evaluation now uses only the held-out test blocks from the seeded split. This alone reduced the real-only baseline from +1.84 to ~+1.25 pp.

(b) Budget confound. Blends were compared at differing total trial counts, confounding data source with data quantity. Fixed-budget mode holds the per-subject total constant across ratios.

(c) Oversampling. An intermediate budget setting sampled with replacement (up to ~8×), inflating every condition. The budget now defaults to each subject's real-pool size, with no replacement.

(d) Normalization confound. The velocity-magnitude feature was normalized by each blend's own statistics at train time but by real statistics at evaluation, giving each ratio a different train/eval offset. A fixed real-derived norm removes this. The effect below not only survives but tightens under fixed norm (paired std ±0.54 → ±0.14).

Minor note: the fixed norm is computed per subject from all real trajectories (two scalars — mean, std). This is identical across ratios, so the ratio comparison is unaffected; it could be tightened to train-only statistics in a future pass.

# 3. Sim calibration (held fixed across ratios)

EEGK-sim is genuine human EEG re-decoded through the current decoder — real within-trajectory texture, but a different global scale and timing than real EEGK. Two distribution-alignment steps bring sim onto real's envelope without rewriting its texture:

- Radius scaling: per-subject origin-scaling so sim's median endpoint radius matches real. Direction-preserving — endpoint classification is unchanged, so scaling cannot manufacture accuracy.

- Dwell alignment: prepend a real-sampled origin-hold so sim reproduces real's "think-then-move" structure. Movement ticks are copied verbatim (texture preserved).

S03/S06 sim is dropped: with no real counterpart, no calibration target exists.

# 4. Hypothesis and protocol

Hypothesis. Because real data is limited per subject, adding calibrated synthetic data to the real set may build a better copilot. At fixed budget this is a composition question: what real:sim mix is best when the total is held constant?

Protocol. For each ratio, build a fixed-budget blend from the training split, train within-subject copilots (fixed norm), and evaluate on the held-out real test split. Repeat over 3 seeds (each reseeds the train/test partition). Report Δaccuracy and Δangle-error vs raw BCI.

# 5. Results — the blend-ratio sweep

Fixed budget, held-out test split, fixed norm, mean ± std over 3 seeds. Δacc = copilot − raw BCI (percentage points); Δerr = angle-error improvement (degrees, + = better).

| Training Ratio (real/sim) | Δacc (pp) | Δerr (°) |  |
| --- | --- | --- | --- |
| real100_sim0 | +1.25 ± 0.33 | −0.08 | ◄ baseline |
| real75_sim25 | +2.14 ± 0.24 | +0.12 | ◄ best |
| real50_sim50 | +0.77 ± 0.51 | −0.01 |  |
| real25_sim75 | +0.90 ± 0.20 | +0.14 |  |
| real0_sim100 | +0.54 ± 0.80 | +0.06 |  |

Reading. real75_sim25 is the best ratio in every seed and beats real-only by +0.89 ± 0.14 pp — a small but highly consistent gain. The curve declines away from the 25%-sim peak, and pure sim is worst. Sim's value is as a minority regularizer, not a data-volume substitute.

Accuracy vs. angle error. The two metrics need not co-optimize: accuracy is a threshold metric (did the endpoint land in the correct 45° wedge?), while angle error is continuous (how many degrees off?). A ratio can win accuracy by nudging borderline trials across the decision boundary, while a different ratio wins angle error by tightening the whole distribution without flipping labels. Here the Δerr differences between blends are small (all within seed-to-seed noise), so we lead on accuracy; the takeaway is only that the accuracy gain does not come at the cost of angular precision.

## Per-subject (3-seed mean)

Δacc in percentage points; Δerr in degrees (+ = better angular precision).

| Subject | Δacc real-only | Δacc 75/25 | Δerr real-only | Δerr 75/25 |
| --- | --- | --- | --- | --- |
| S01 | +0.62 | +1.85 | +0.08 | +0.15 |
| S02 | +2.71 | +3.83 | +0.34 | +0.48 |
| S04 | +2.62 | +2.36 | −0.72 | −0.85 |
| S05 | +0.70 | +1.29 | −0.16 | +0.06 |
| S07 | −2.63 | −0.58 | −0.60 | +0.25 |

The blend helps four of five subjects on accuracy and is roughly neutral on S04. Its largest effect is on S07 — the sparsest (114 test trials), most chaotic subject — where it more than halves the accuracy damage (−2.63 → −0.58) and flips angle error from −0.60° to +0.25°, though S07 remains net-negative on accuracy. S07 also drives most of the seed-to-seed variance; the overall gain is carried by the reliably-correctable subjects. Note S04 is the one subject where the blend slightly worsens angular precision (−0.85° vs −0.72°) — the only per-subject sign that more sim can hurt.

# 6. Is synthetic augmentation worth extending? A residual-error diagnostic

The copilot's job is to rescue trials the raw BCI gets wrong — nudge trajectories that would miss into the correct wedge. So the relevant universe is the BCI-wrong trials (~378 across the test set): the only ones there is anything to fix. A surrogate would be worth building only if it could rescue trials that calibrated sim cannot. Sim already carries real EEG texture, so its one plausible weakness versus a purpose-built surrogate is exact global structure — in particular, matching each subject's chaotic/high-reversal trials (the wild trajectories, especially S07's). The surrogate's pitch is therefore: "model that chaos exactly and rescue the chaotic trials sim leaves behind."

For that pitch to hold, the trials the blend still gets wrong must be disproportionately the chaotic ones — a structured, chaos-shaped gap. We tested this directly: roll the real-only and 75/25 models on the held-out test split, and split the still-wrong trials by a chaos index (trajectory path length ÷ straight-line distance; high = wandered/reversed a lot). Then compare the chaos level of the still-wrong trials against the correctable pool as a whole.

| Diagnostic (3-seed) | Finding |
| --- | --- |
| Headroom | Of ~378 BCI-wrong trials, the blend fixes only ~13%; ~87% remain wrong. |
| Sim's marginal edge | Trials the blend fixes but real-only misses are NOT reliably more chaotic (38% / 20% / 31% vs 34% baseline — flips across seeds). |
| Residual character | Trials still wrong after the blend are NOT enriched in chaos (33.2 / 34.1 / 33.8% vs 34.4% baseline) — ordinary hard trials, every seed. |

Interpretation. Two things follow. First, the still-wrong trials are not a chaotic subset — their chaos level matches the correctable pool at large — so there is no chaos-shaped gap for a surrogate's exact structure-modeling to target. Second, the blend rescues only ~13% of correctable trials; the ~87% that stay wrong do so because the decoder produced no recoverable intent in the trajectory (the cursor went nowhere near the target, with no correctable signal). No copilot, trained on any data, can rescue a trajectory that carries no information about the intended target. That ceiling is a decoder limit, not a training-data limit.

Recommendation. Do not build the process-modeled surrogate on the current rationale. The binding constraint on copilot headroom is decoder-level correctability, not the texture/structure gap between real and synthetic training data.

# 7. Status and next steps

Where this leaves the question. Measured cleanly, calibrated EEGK-sim is a useful minority augmenter: a 75/25 real/sim blend beats real-only by ~0.9 pp, robustly. This reverses the prior report's "synthetic does not beat real" — which was measured under evaluation leakage and an uncontrolled budget.

- Extend to 5 seeds. 3 seeds already give a tight ±0.14 pp on the key contrast; 5 would support a formal confidence interval. (Polish, not a blocker.)

- Surrogate: deprioritize. The diagnostic argues against the structure-gap rationale. If pursued, frame it as confirming a tested-unlikely mechanism, not an expected win.

- Investigate the correctability ceiling. The ~87% uncorrectable rate is the real lever. Whether it is a decoder limit or a subset of recoverable-but-missed trials is the more consequential question than synthetic-data choice.

- S07. Remains net-negative under every condition; its chaos and sparsity make it the dominant source of variance. Worth deciding whether it is signal to match or an outlier to set aside.